ABB AO895 3BSC690087R1 Analog Output Module

ABB AO895 3BSC690087R1 is a Analog Output Module, it has has 8 channels and includes Intrinsic Safety protection components and a HART interface on each channel for connection to process equipment in hazardous areas without the need for additional external devices.

Brand: ABB

Module Number: AO895

Order Number: 3BSC690087R1

Price: $1000

Country Of Origin: Sweden and Switzerland

Weight: 0.35KG

Certificate: C/O from the Chamber of Commerce<br>C/Q from the manufacturer

Warranty: 12 Months

Inventory Qty: 2

Description:

ABB AO895 Key parameters

| Parameter | Specification |
| --- | --- |
| Signal specification | 4...20 mA |
| Number of channels | 8 |
| Signal type | Unipolar single ended |
| Dielectric test voltage | 500 V a.c. |
| Mechanics | S800 |

一．ABB AO895 Features and benefits

8 channels of 4...20 mA outputs.

HART communication.

1 group of 8 channels isolated from ground.

Power to drive Ex certified I/P actuators.

The AO895 Analog Output Module has 8 channels. The module includes Intrinsic Safety protection components and a HART interface on each channel for connection to process equipment in hazardous areas without the need for additional external devices.

Each channel can drive up to 20 mA loop current into a field load such as an Ex-certified current-to-pressure converter and is limited to 22 mA in overload conditions. All eight channels are isolated from the ModuleBus and power supply in one group. Power to the output stages is converted from the 24 V on the power supply connections.