ABB DI840 3BSE020836R1 Digital Input Module

ABB DI840 3BSE020836R1 is a Digital Input Module, This product is used across various industrial sectors as a fundamental data input module, yet it plays a crucial role.

Brand: ABB

Module Number: DI840

Order Number: 3BSE020836R1

Price: $700

Country Of Origin: Sweden and Switzerland

Weight: 0.35KG

Certificate: C/O from the Chamber of Commerce<br>C/Q from the manufacturer

Warranty: 12 Months

Inventory Qty: 1

Description:

ABB DI840 Key parameters

| Parameter | Specification |
| --- | --- |
| Article number | 3BSE020836R1 |
| Input voltage range, "0" / "1" | -30..+5 V / 11..30 V |
| Input impedance | 3.3 kΩ |
| Power dissipation | Typ. 2.7 W |
| Current consumption +5 V Modulebus | 100 mA |

一．ABB DI840 Features and Benefits

16 channels for 24 V d.c. inputs with current sinking

1 group of 16 isolated from ground

Input status indicators

Advanced on-board diagnostics

Sequence of events

Redundant or single applications

Transducer power distribution

More information

The DI840 is a 16 channel 24 V d.c. digital input module for single or redundant applications. The input voltage range is 18 to 30 V d.c. and the input current is 7 mA at 24 V d.c. Each input channel consists of current limiting components, EMC protection components, input state indication LED and optical isolation barrier.

The transducer power is supervised and current limited; one output per two input channels. The Sequence of Event function (SOE) can collect events with a resolution of 1 ms. The event queue can contain up to 257 events. The function includes a Shutter filter for filtering out unwanted events.

ABB DI840 compared with DI830

| Module Model | DI830 | DI840 |
| --- | --- | --- |
| Brand | ABB | ABB |
| Primary Type | 16-channel digital input module | Digital input module / Pulse digital counter module |
| Core Function | Receives external digital signals | Receives digital signals or counts/measures pulses |